# ARRURU SATHISH REPORT

Real-Time Temperature Monitoring Using Arduino

1. Introduction

This project is a simple temperature monitoring system using Arduino.
A temperature sensor reads the temperature, and based on the value, two LEDs and a buzzer are controlled.
If the temperature is high, the red LED and buzzer turn ON.
If the temperature is normal, the blue/green LED turns ON.

2. Components Used

- Arduino Uno

- Temperature Sensor (LM35 / TMP36)

- 2 LEDs (Red and Blue/Green)

- Buzzer

- 2 × 220Ω resistors

- Connecting wires

- USB cable

3. Circuit Connections

Temperature Sensor

- Left pin → 5V

- Middle pin → A0

- Right pin → GND

Cool LED (Blue/Green)

- Pin 8 → 220Ω resistor → LED +

- LED – → GND

Hot LED (Red)

- Pin 9 → 220Ω resistor → LED +

- LED – → GND

Buzzer

- Pin 10 → Buzzer +

- Buzzer – → GND

4. Working

- Arduino reads the temperature from the sensor.

- The value is converted to °C.

- If the temperature is greater than the threshold,

  - Red LED turns ON

  - Buzzer turns ON

  - Blue LED turns OFF

- If the temperature is below or equal to the threshold,

  - Blue LED turns ON

  - Red LED and buzzer turn OFF

The system works continuously in real time.

5. Code

const int sensorPin = A0;

const int coolLed = 8;

const int hotLed = 9;

const int buzzer = 10;

float threshold = 35.0;

void setup() {

pinMode(coolLed, OUTPUT);

pinMode(hotLed, OUTPUT);

pinMode(buzzer, OUTPUT);

Serial.begin(9600);

}

void loop() {

int sensorValue = analogRead(sensorPin);

float voltage = sensorValue * (5.0 / 1023.0);

float temperature = voltage * 100.0;

if (temperature > threshold) {

digitalWrite(hotLed, HIGH);

digitalWrite(buzzer, HIGH);

digitalWrite(coolLed, LOW);

} else {

digitalWrite(coolLed, HIGH);

digitalWrite(hotLed, LOW);

digitalWrite(buzzer, LOW);

}

delay(200);

}

6. Applications

- Home temperature alert

- Basic safety alarm

- School/college mini projects

- Server/room temperature monitoring

7. Conclusion

The project successfully detects temperature in real time and activates LEDs and a buzzer based on the threshold. It is simple, low-cost, and useful for learning Arduino and sensor interfacing.